###### Chapter 03.03

###### Bisection Method of Solving a Nonlinear Equation – More Examples

###### Electrical Engineering

# Example 1

Thermistors are temperature-measuring devices based on the principle that the thermistor material exhibits a change in electrical resistance with a change in temperature.  By measuring the resistance of the thermistor material, one can then determine the temperature.

For a 10K3A Betatherm thermistor,

Thermally conductive epoxy coating Tin plated copper alloy lead wires

Figure 1 A typical thermistor.

the relationship between the resistance R of the thermistor and the temperature is given by

1T=1.129241×10−3+2.341077×10−4ln(R)+8.775468×10−8lnR3

where T is in Kelvin and R is in ohms.

A thermistor error of no more than ±0.01 °C is acceptable. To find the range of the resistance that is within this acceptable limit at 19°C, we need to solve

119.01+273.15=1.129241×10−3+2.341077×10−4ln(R)+8.775468×10−8lnR3

and

118.99+273.15=1.129241×10−3+2.341077×10−4ln(R)+8.775468×10−8lnR3

Use the bisection method of finding roots of equations to find the resistance R at 18.99 °C.  Conduct three iterations to estimate the root of the above equation. Find the absolute relative approximate error at the end of each iteration and the number of significant digits at least correct at the end of each iteration.

# Solution

Solving

118.99+273.15=1.129241×10−3+2.341077×10−4ln(R)+8.775468×10−8lnR3

we get

f(R)=2.341077×10−4ln(R)+8.775468×10−8lnR3−2.293775×10−3

Lets us assume

Rℓ=11000, Ru=14000

Check if the function changes sign between Rℓ and Ru.

f(Rℓ)=f(11000) =2.341077×10−4ln(11000)+8.775468×10−8ln110003 −2.293775×10−3 =−4.4536×10−5  f(Ru)=f(14000) =2.341077×10−4ln(14000)+8.775468×10−8ln140003 −2.293775×10−3 =1.7563×10−5

Hence

fRℓfRu=f11000f14000=−4.4536×10−51.7563×10−5<0

So there is at least one root between Rℓ and Ru, that is, between 11000 and 14000.

## Iteration 1

The estimate of the root is

Rm=Rℓ+Ru2

=11000+140002

=12500

f(Rm)=f(12500) =2.341077×10−4ln(12500)+8.775468×10−8ln125003−2.293775×10−3 =−1.1655×10−5fRℓfRm=f11000f12500=−4.4536×10−5−1.1655×10−5>0

Hence the root is bracketed between Rm and Ru, that is, between 12500 and 14000. So, the lower and upper limits of the new bracket are

Rℓ=12500, Ru=14000

At this point, the absolute relative approximate error ∈a cannot be calculated as we do not have a previous approximation.

## Iteration 2

The estimate of the root is

Rm=Rℓ+Ru2

=12500+140002

=13250

fRm=f13250 =2.341077×10−4ln13250+8.775468×10−8ln132503−2.293775×10−3 =3.3599×10−6fRℓfRm=f12500f13250=−1.1655×10−53.3599×10−6<0

Hence, the root is bracketed between Rℓ and Rm, that is, between 12500 and 13250.

So the lower and upper limits of the new bracket are

Rℓ=12500, Ru=13250

The absolute relative approximate error ∈a at the end of Iteration 2 is

∈a=Rmnew−RmoldRmnew×100

=13250−1250013250×100

=5.6604%

None of the significant digits are at least correct in the estimated root

Rm=13250

as the absolute relative approximate error is greater than 5%.

## Iteration 3

Rm=Rℓ+Ru2

=12500+132502

=12875

fRm=f12875 =2.341077×10−4ln12875+8.775468×10−8ln128753−2.293775×10−3 =−4.0403×10−6	fRℓfRm=f12500f12875=−1.1654×10−5−4.0398×10−6>0

Hence, the root is bracketed between Rmand Ru, that is, between 12875 and 13250.

So, the lower and upper limits of the new bracket are

Rℓ=12875, Ru=13250

The absolute relative approximate error ∈a at the end of Iteration 3 is

∈a=Rmnew−RmoldRmnew×100

=12875−1325012875×100

=2.9126%

One of the significant digits is at least correct in the estimated root of the equation as the absolute relative approximate error is less than 5%.

Seven more iterations were conducted and these iterations are shown in the Table 1.

Table 1 Root of fx=0 as a function of the number of iterations for bisection method. Iteration; Rℓ; Ru; Rm; ∈a%; fRm. 1 2 3 4 5 6 7 8 9 10; 11000 12500 12500 12875 13063 13063 13063 13063 13074 13074; 14000 14000 13250 13250 13250 13156 13109 13086 13086 13080; 12500 13250 12875 13063 13156 13109 13086 13074 13080 13077; ---------- 5.6604 2.9126 1.4354 0.71259 0.35757 0.17910 0.089633 0.044796 0.022403; 1.1655×10−5 3.3599×10−6 −4.0403×10−6 −3.1417×10−7 1.5293×10−6 6.0917×10−7 1.4791×10−7 −8.3022×10−8 3.2470×10−8 −2.5270×10−8.

At the end of the10th iteration,

∈a=0.022403 %

Hence the number of significant digits at least correct is given by the largest value of m for which

∈a≤0.5×102−m

0.022403≤0.5×102−m

0.044806≤102−m

log0.044806≤2−m

m≤2−log0.044806=3.3487

So

m=3

The number of significant digits at least correct in the estimated root 13077 is 3.

| NONLINEAR EQUATIONS |  |  |
| --- | --- | --- |
| Topic | Bisection Method-More Examples |  |
| Summary | Examples of Bisection Method |  |
| Major | Electrical Engineering |  |
| Authors | Autar Kaw |  |
| Date | July 30, 2026 |  |
| Web Site | http://numericalmethods.eng.usf.edu |  |